+10

You can use Choreo (the constraint-honoring trajectory planner and editor) alongside WPILib to program autonomous path-following for a Romi differential drive robot. Because Romi uses a differential (tank) drive configuration [12.11], you can test time-optimal paths right on your desktop using simulation [12.11] before running them on the physical hardware. [1, 2, 3, 4, 5, 6]  Open your Romi 2026 branch in VSCode and install the vendor dependency Choreo.

https://choreo.autos/lib/ChoreoLib2026.json

Getting Started with Choreo and WPILib

- Download and open the desktop application from Choreo Documentation.

- https://docs.wpilib.org/en/stable/docs/software/pathplanning/choreo/index.html

- Create a new project configured for a differential drivebase rather than swerve.

- Input your Romi's physical constraints (track width, max velocity, and max acceleration) into the profile settings. [1, 2, 3]

Exporting and Project Setup

- Draw your desired autonomous path using waypoints and constraints inside the Choreo UI.

- Configure code generation to output trajectory files directly into your WPILib project directory (e.g., src/main/deploy/choreo or equivalent Java/C++ source folders).

- Ensure your Romi project is set up using the standard WPILib VS Code extension. [1, 2, 3, 4, 5]

Simulating and Running on Romi

- Build your robot code containing the path-following logic using ChoreoLib or WPILib trajectory utilities.

- Power on your Romi and connect your computer to its broadcasted WPILibPi-<number> Wi-Fi network [12.11].

- Open the WPILib Command Palette (Ctrl + Shift + P) in VS Code and launch Simulate Robot Code (F5) to watch your desktop code drive the physical Romi through your Choreo-optimized path. [1, 2, 3, 4, 5]

Watch this video to understand the layout and structure of a WPILib Romi example project:

8m

Romi WPILib Example Project Walkthrough

2.5K views · 5 years ago

YouTube · Spectrum3847